EBONE

-----

EBONE is a European international network backbone connecting research

network service providers. Connectivity will be offered between these

networks, to the United States and indeed to the International Internet.

EBONE will offer two services:

- an Internet IP (level 3) production service;

- an ISO-CLNS (level3 ) pilot service.

EBONE is constituted as a consortium of organizations which contribute to

the management, operation, and funding of the network.  'EBONE' stands for

"European Backbone".

The Need for a European Research Backbone Network

The requirement for Europe-wide network connectivity at increasing

bandwidth and for high-speed connectivity to US networks has long been

recognized.  In recent years, this demand has focused on IP services.

Until late 1991 such connectivity was available only to individual

national and international research networks.  With EBONE, network

interconnection is simplified, increased bandwidth is made available,

better connections are provided to the United States, and there is greater

economy of scale in terms of operations and transmission cost.

The Community Served

EBONE focuses on supporting networking organizations which serve the

European academic and research communities.  Through EBONE, European

researchers have improved access and higher-performance connections to

their colleagues throughout Europe and the United States.

Furthermore, by encouraging the participation of commercial network

service providers (e.g., PTTs, information technology companies), EBONE

will increase the size of the participating communities, reduce individual

costs, encourage the participation of industrial researchers, and

stimulate the creation of competitive international IP networking services

in Europe.

EBONE complements other European activities such as the X.25 service

provided by the IXI network, and the EMPP pilot multi-protocol project.

EBONE Today

EBONE will operate a core backbone between London, Stockholm, Amsterdam,

Geneva, and France (Montpellier).

Stockholm  E

/   \

256 Kbps /       \ 512 Kbps

/           \

/               \

London E                 E Amsterdam

|                 |

|        512 Kbps |

|                 |

|                 |

| 256 Kbps        |

|                 |

France (Montpellier) E-----------------E CERN

256 Kbps

Intercontinental links to the United States are provided from London,

Stockholm and Geneva.  The European links operate initially at speeds

between 256 and 512 kilobits per second.  National and international

networks connect to these core backbone sites.  Demand for EBONE services

is growing rapidly.  Additional links and increased capacity will be

installed to satisfy this demand, and several such upgrades are in

planning.

EBONE is managed by the EBONE Management Committee made up of

representatives from selected member organizations.  Operational support

is provided by core and other sites in cooperation with EBONE Network

Operations Center at The Royal Institute of Technology (KTH) in Stockholm.

Background and Time Scales

EBONE began in September 1991 when representatives of several European

academic and research networks met to resolve long-standing European

connectivity problems.  Their approach was to evaluate existing available

links, to look for opportunities to bring these links together quickly

under a unified approach, and to make plans to enhance these links.

Contributions were secured, a management structure was established,

operational procedures were put in place, and an overall

contribution-oriented funding approach was agreed.

The effort was formally started in January 1992.  Each participating

organization has signed a Memorandum of Understanding which defines the

terms of EBONE membership and the resources which each member contributes

to the EBONE effort.  EBONE now has 20 member organizations (see below),

with new members joining regularly.

The present flexible contribution-oriented approach is expected to evolve

towards a more formal funding structure.  Over the longer term, it is

anticipated that the EBONE will become a part of the multi-protocol

networking infrastructure to be provided by the RARE initiated Operational

Unit.

For More Information About EBONE

Please contact Marieke Dekker at the RARE Secretariat (E-mail:

ebone@rare.nl, phone: +31 20 639 1131) or any of the participating

organizations listed.

ANNEX

The following Member Organizations have formally joined the EBONE

Consortium.

ACOnet, Austria

ARIADNet, Greece

EARN

EUnet/EurOpen

FORTH, Greece (Crete)

ICRF, UK

ILAN, Israel

JNT, UK

KTH, Sweden

KU Leuven, Belgium

NIKHEF, Netherlands

Nordic Carriers

NORDUnet

RARE

RedIRIS, Spain

Swedish Telecom International/TIPnet

SURFnet, Netherlands

SWIPnet/Tele2, Sweden

ULB, Belgium

YUNAC, Yugoslavia

The following organizations have formally expressed support and

cooperation with the EBONE initiative, while not intending to join the

Consortium by signing the MoU:

CERN

IBM

NSF

PTT Telecom, Netherlands

The following organizations have expressed interest in joining the

EBONE Consortium and announced the intention of signing the MoU:

DFN, Germany

FCCN, Portugal

GMD, Germany

RENATER, France

SWITCH, Switzerland

UCD, Ireland

XLINK, Germany

Other organizations having shown interest in joining the Consortium:

ECRC

ESA

INFN/GARR, Italy

INRIA, France

HEANET, Ireland

Rutherford laboratories, UK

GLOSSARY

CLNS:  Connection-less network service

EMPP:  European 2 Mb MultiProtocol Pilot

IP:    Internet Protocol

ISO:   International Standards Organization

IXI:   International X.25 Infrastructure

RARE:  Reseaux Associes pour la Recherche Europeenne